# PULSE Instrument Cluster

Requirements Traceability Matrix

Generated by scripts/gen_rtm.py

2026-10-02

Table of Contents

# Document control

| Field | Value |
| --- | --- |
| Document ID | PULSE-RTM-001 |
| Title | Requirements Traceability Matrix |
| Generated | 2026-10-02 by scripts/gen_rtm.py |
| Status | Derived; regenerate, never hand-edit |
| Sources | PULSE-StRS-001, PULSE-SyRS-001, PULSE-SAD-001, PULSE-SAF-001, PULSE-SEC-001 |

This matrix is produced from the ID tables in the source documents. A requirement’s status, verification method and evidence are copied from its owning document. A story’s trace list is the only link authored by hand. Dangling references fail the generator, so a matrix that exists is a matrix that is consistent.

# 1. Status summary

| Group | Implemented | Partial | Planned S2 | Planned S3 | Planned S4 | Total |
| --- | --- | --- | --- | --- | --- | --- |
| Functional (FR) | 16 | 4 | 0 | 1 | 1 | 22 |
| Interface (IR) | 5 | 0 | 0 | 0 | 0 | 5 |
| Non-functional (NFR) | 4 | 2 | 0 | 2 | 0 | 8 |
| Functional safety (SR) | 7 | 2 | 0 | 1 | 1 | 11 |
| Cybersecurity (CS) | 8 | 1 | 0 | 1 | 1 | 11 |
| All | 40 | 9 | 0 | 5 | 3 | 57 |

Coverage: 36 of 57 requirements are reached by at least one user story; 21 are traced only through their owning document (listed in section 5).

# 2. Stakeholder needs to stories to requirements

| Need | Statement | Stories | Requirements reached |
| --- | --- | --- | --- |
| SN-1 | Decide the cluster toolkit on measured evidence, with application cost separated from platform cost | US-5, US-8, US-10 | FR-7, FR-12, FR-13, FR-15, FR-16, FR-17, NFR-3, NFR-7 |
| SN-2 | Build in-house competence in the hybrid cockpit pattern: one broker, a Linux cluster, an Android surface | US-2, US-8 | FR-2, FR-3, FR-12, FR-13 |
| SN-3 | An HMI decoupled from the vehicle bus so a platform change does not restart HMI work | US-1, US-3 | FR-1, FR-5, FR-8, FR-11 |
| SN-4 | A demonstrator presentable to a non-technical audience in under five minutes | US-4, US-6, US-7, US-9 | FR-6, FR-9, FR-10, FR-14, IR-3, IR-4, NFR-2 |
| SN-5 | Competence that outlives the project: a new engineer brings the stack up from documentation alone | US-11 | NFR-5 |
| SN-6 | Compliance readiness: the safety and security artefacts a production programme would ask for, produced while building rather than retrofitted | US-14, US-15, US-16, US-17 | CS-1, CS-2, CS-3, CS-6, SR-1, SR-2, SR-3, SR-9, SR-11 |
| SN-7 | A closed loop, not a dashboard light: perception feeds a vehicle reaction through the same signal path the cluster renders | US-12, US-13 | FR-18, FR-19, FR-20, FR-21, SR-7 |

# 3. Requirement to story, verification, status and evidence

## 3.1 Functional (FR), owner PULSE-SyRS-001

| ID | Requirement | Stories | Verif. | Status (W3) | Evidence |
| --- | --- | --- | --- | --- | --- |
| FR-1 | The system shall represent all vehicle data using an open, standardised signal catalogue (COVESA VSS), not proprietary or bus-specific identifiers. | US-1 | I | Implemented | vss/vss_agl.json compiled from VSS 6.0; 1,302 leaf signals |
| FR-2 | The system shall extend the standard catalogue for domain-specific signals using the standard overlay mechanism only, with no forks of the upstream specification. | US-2 | I | Implemented | vss/agl_vss_overlay.vspec, vss/pulse_vss_overlay.vspec, vss/dms_vss_overlay.vspec; upstream spec consumed unmodified |
| FR-3 | The compiled signal tree shall be reproducible from versioned source files by a documented command. | US-2 | T | Implemented | Suite check vss-regeneration, byte-identical with vss-tools 6.0 in .venv-tools; first executed 2026-09-23, it had skipped before for want of the tool |
| FR-4 | A central broker shall serve the signal tree and allow multiple independent clients to subscribe concurrently. | none | D | Implemented | KUKSA databroker 0.7.0; Flutter cluster, Compose cluster and engine-audio service subscribe concurrently; suite check broker-up |
| FR-5 | Signal producers and consumers shall be mutually decoupled: neither needs knowledge of the other’s implementation. | US-3 | I | Implemented | Providers and HMIs share no code, only VSS paths (PULSE-SAD-001 section 4) |
| FR-6 | The system shall provide a telemetry source that generates physically plausible, correlated vehicle behaviour (speed, engine speed, gear, temperature, fuel, odometer) without any vehicle hardware. | US-4 | D | Implemented | emulator/telemetry_sim.py: lap and cruise cycles with correlated RPM, fuel and odometer. Suite check live-telemetry passes |
| FR-7 | The telemetry source shall run a repeatable cycle so that UI behaviour can be compared across runs and across implementations. | US-5 | T | Partial | Phase table is fixed with no randomness, but sampling is wall-clock driven, so runs differ in timing. Fixed-step replay in S4 |
| FR-8 | The telemetry source shall be replaceable by a real vehicle-bus provider without modification to any HMI code. | US-3 | T | Partial | Proven on a virtual bus on 23 September (scripts/can_spike.py). Since 2 October the simulator can act as the vehicle’s ECUs (SIM_CAN=vcan0… |
| FR-9 | The HMI shall display, updating live: road speed, engine speed with warning and redline indication, selected gear, fuel level and range, per-corner tyre pressures, and driver-relevant status… | US-6 | D | Implemented | pulse-cluster/lib/screen/pulse_screen.dart; subscribed paths listed in PULSE-SAD-001 appendix A |
| FR-10 | The HMI shall display motorsport telemetry: current lap number, current and best lap time, aerodynamic (DRS) state, and energy-recovery (ERS) level and state. | US-7 | D | Implemented | Vehicle.Motorsport.* overlay signals rendered by both HMIs |
| FR-11 | The HMI shall subscribe to abstract signals only. It shall not contain CAN frame parsing, DBC knowledge, or hardware-specific decoding. | US-1 | T | Implemented | Regression suite check hmi-purity scans both HMI sources for CAN, DBC and SocketCAN tokens; passing, report in docs/evidence/ |
| FR-12 | The HMI shall be implemented twice, once in Flutter for Linux and once in Compose for Android Automotive, from a single shared visual design. | US-8 | I | Implemented | pulse-cluster/ (Flutter) and pulse-cluster-android/ (Compose); design source in design/. Both seen running on 2 October, Compose on the Android 16 car… |
| FR-13 | Both implementations shall consume the identical signal source, so any observed difference is attributable to the platform, not the data. | US-8 | D | Implemented | Since 2 October both clusters subscribe to the same 28 paths through the same broker, protocol, catalogue limits and freshness rules. The benchmark figures in… |
| FR-14 | The HMI shall render full-screen on the target strip display, and shall degrade gracefully to a normal window when that display is absent. | US-9 | D | Implemented | run.sh locates a 2560 x 720 output at runtime and passes CLUSTER_GEOM; unset means a normal window |
| FR-15 | The system shall support measurement of both implementations under identical telemetry: CPU utilisation, memory footprint, and shipped artefact size. | US-10 | T | Partial | Measured once by hand: 123 vs 46 MB memory, 12.2 vs 11.5 % CPU, 50 vs 23 MB artefact (BRIEF.md). Scripted benchmark in S4 |
| FR-16 | Measurements shall distinguish application-level cost from platform-level cost, since these drive different procurement decisions. | US-10 | A | Partial | Platform memory reported separately (AAOS 3.2 to 4.0 GB from emulator); target-hardware figures planned S3 to S4 |
| FR-17 | Results shall be documented such that a reviewer can reproduce them from the repository. | US-10 | T | Planned S4 | Benchmark script and results file to be committed under docs/ |
| FR-18 | The system shall derive driver fatigue and distraction levels from a driver-facing camera using inference executed entirely on the local device, and publish them as VSS driver signals. | US-12 | D | Implemented | emulator/dms.py: MediaPipe FaceLandmarker on CPU; publishes Vehicle.Driver.FatigueLevel, DistractionLevel, AttentiveProbability, IsEyesOnRoad at 10 Hz |
| FR-19 | The system shall maintain a driver alert state of NONE, DROWSY, DISTRACTED or NO_FACE. It shall raise an alert only after a configurable hold time, and release it only after a configurable clear… | US-12 | T | Implemented | DMS_ALERT_HOLD_S 3.0 s, DMS_ALERT_CLEAR_S 2.0 s; published as Vehicle.Driver.Monitoring.AlertState |
| FR-20 | When cruise mode is active and the alert state is DROWSY, the vehicle model shall execute the minimum-risk manoeuvre specified in PULSE-SAF-001 SM-3, which the driver can abort by recovering… | US-13 | T | Implemented | telemetry_sim.py cruise mode implements SM-3; published as Vehicle.Driver.Monitoring.InterventionState and InterventionCountdown |
| FR-21 | The HMI shall render the driver attention level, the active alert as a full-width banner distinguishable by colour per alert type, and the intervention countdown. | US-12 | D | Implemented | Attention panel and banner in pulse_screen.dart; attention meter in classic_screen.dart |
| FR-22 | The system shall accept an autonomy-stack provider that publishes planned trajectory, detected objects and drive mode into the broker as VSS signals, so that the cluster can render an autonomy state… | none | D | Planned S3 | Autoware planning-simulation bridge, work package 04; VSS overlay for autonomy signals to be added under vss/ |

## 3.2 Interface (IR), owner PULSE-SyRS-001

| ID | Requirement | Stories | Verif. | Status (W3) | Evidence |
| --- | --- | --- | --- | --- | --- |
| IR-1 | Client-to-broker communication shall use a language-neutral RPC interface supporting streaming subscriptions. | none | I | Implemented | gRPC over HTTP/2, kuksa.val.v1 Subscribe stream; Dart and Kotlin stubs generated from the same .proto |
| IR-2 | Signal semantics (units, ranges, allowed enumeration values, and encodings) shall be defined in the signal catalogue, not in client code. | none | I | Implemented | Units, allowed lists and max in the .vspec overlays; broker rejects out-of-catalogue values on set; suite check unit-sanity catches a value published… |
| IR-3 | The target display interface shall be a 2560 x 720 automotive strip panel. | US-9 | D | Implemented | Corsair Xeneon Edge on the desk rig; layout designed at 2560 x 720 |
| IR-4 | Display placement shall be configurable at runtime, not fixed at compilation. | US-9 | D | Implemented | CLUSTER_GEOM environment variable read by the Linux runner; run.sh derives it from xrandr |
| IR-5 | The system shall run without transport security in the local development configuration, and the security posture shall be an explicit, documented choice. | none | I | Implemented | Secure by default. Plaintext only with PULSE_INSECURE=1, which prints a boxed warning at start-up; suite check transport-security skips and says so |

## 3.3 Non-functional (NFR), owner PULSE-SyRS-001

| ID | Requirement | Stories | Verif. | Status (W3) | Evidence |
| --- | --- | --- | --- | --- | --- |
| NFR-1 | Responsiveness: displayed values shall track the telemetry source with no perceptible lag at a glance. | none | D | Implemented | Fast signals at 10 Hz, one broker tick from publish to render; observed at the review demo |
| NFR-2 | Legibility: primary values (speed, gear, engine speed) shall be readable in a sub-second glance, consistent with driver-distraction practice. | US-6 | D | Implemented | Design reviewed on the strip; numerals sized for the 720 px height |
| NFR-3 | Footprint: the Linux implementation shall be deployable on constrained embedded hardware; total platform footprint is a first-class evaluation criterion. | US-10 | T | Planned S3 | Raspberry Pi 5 baseline, work package 03, week 9 |
| NFR-4 | Portability: migration to a different SoC or to a Yocto/AGL image shall not require HMI rework. | none | A | Planned S3 | Argument to be recorded with the Pi 5 baseline; Flutter embedder is the only platform-specific layer |
| NFR-5 | Reproducibility: a new engineer shall be able to bring up the full stack on a clean machine from documentation alone. | US-11 | T | Partial | Bring-up of 2026-08-18 in ONBOARDING.md. On 2026-10-02 a second host rebuilt both environments from requirements.txt with constraints.txt and ran every… |
| NFR-6 | Openness: the stack shall be built from open-source components, with no proprietary runtime licence required for evaluation. | none | I | Implemented | KUKSA (Apache-2.0), VSS (MPL-2.0), Flutter (BSD), MediaPipe (Apache-2.0), AOSP; NOTICE file for the face model |
| NFR-7 | Determinism: repeated runs of the same cycle shall produce the same signal sequence, so comparisons are valid. | US-5 | T | Partial | Same as FR-7: value sequence is deterministic in shape, sample timing is not. Fixed-step replay planned S4 |
| NFR-8 | Maintainability: upstream components shall be consumed at pinned versions, not floating heads. | none | I | Implemented | Broker by tag and digest; Python by requirements.txt on Python 3.11; VSS 6.0 and vss-tools 6.0; Dart by pubspec.lock; Flutter 3.47.0 in .fvmrc, enforced… |

## 3.4 Functional safety (SR), owner PULSE-SAF-001

| ID | Requirement | Stories | Verif. | Status (W3) | Evidence |
| --- | --- | --- | --- | --- | --- |
| SR-1 | An item definition shall be written for the cluster and driver-monitoring function, naming the system boundary, the assumed vehicle context, and the interfaces crossing that boundary (ISO 26262-3). | US-15 | I | Partial | Section 2 of this document |
| SR-2 | A preliminary hazard analysis and risk assessment (HARA) shall be recorded for the displayed functions, with severity, exposure and controllability rated per malfunction and a resulting ASIL proposal. | US-15 | I | Partial | Section 4; eight hazards rated; review in S2 |
| SR-3 | Signals whose corruption or loss could mislead the driver (road speed, gear, warning indicators, drowsiness alert) shall be identified as safety-relevant and listed explicitly. | US-15 | I | Implemented | Section 3 |
| SR-4 | The system shall detect loss of a safety-relevant signal, including broker disconnection and stale values, and shall indicate the degraded state rather than continuing to display the last known value. | none | T | Implemented | SM-1. Flutter: pulse-cluster/test/cluster_smoke_test.dart, arrival_test.dart, and captures on the physical strip live, one second after the broker was… |
| SR-5 | Every safety-relevant signal shall carry a freshness criterion; a value older than its criterion shall be treated as invalid. | none | T | Implemented | Criteria in section 3 are the kFreshness table in vehicle_state.dart and FRESHNESS_NS in VssClient.kt, all eight in both since 2 October; providers… |
| SR-6 | Safety-relevant display elements shall fail visibly. A rendering fault shall not present a plausible but wrong value. | none | T | Planned S3 | SM-2 |
| SR-7 | The minimum-risk manoeuvre triggered by sustained driver drowsiness shall be specified as a safety mechanism: trigger condition, driver-recovery window, escalation, and abort condition. | US-13 | D | Implemented | SM-3; implemented in emulator/telemetry_sim.py cruise mode; demonstrated live |
| SR-8 | Freedom from interference shall be argued for the mixed-criticality cockpit: infotainment content shall not be able to degrade or obscure the safety-relevant cluster surface. | none | A | Planned S4 | Section 6 outline |
| SR-9 | A safety-relevant requirement shall be traceable to the verification that demonstrates it, and each verification result shall be reproducible from the repository. | US-17 | T | Implemented | PULSE-RTM-001 generated from this table; 16-check suite runs before each demo and in CI, report committed to docs/evidence/; SR-4 and SR-5 have named tests |
| SR-10 | Any decision that would block a later ASIL argument, such as the choice of graphics stack, operating system or toolkit, shall be recorded as a documented deviation with its rationale. | none | I | Implemented | PULSE-SAD-001 section 8, DD-1 to DD-11 |
| SR-11 | A qualification gap list shall be maintained: for each component, what a production programme would still need (qualified toolchain, certified OS, evidence of a safety-qualified renderer). | US-15 | I | Implemented | Section 8 |

## 3.5 Cybersecurity (CS), owner PULSE-SEC-001

| ID | Requirement | Stories | Verif. | Status (W3) | Evidence |
| --- | --- | --- | --- | --- | --- |
| CS-1 | A threat analysis and risk assessment (TARA) shall be recorded for the signal path, covering the broker, both HMI clients, the telemetry provider and the camera input. Each threat scenario shall… | US-16 | I | Partial | Section 4, nine scenarios |
| CS-2 | The item boundary and its trust boundaries shall be documented, distinguishing on-device interfaces from anything reachable off the device. | US-16 | I | Implemented | Section 2 |
| CS-3 | Broker communication shall support authenticated, encrypted transport (mutual TLS). Running without it shall be a configuration reserved for local development and flagged at start-up. | US-16 | T | Implemented | TLS with a development certificate and a signed token per client, secure by default in scripts/start-databroker.sh; plaintext only with PULSE_INSECURE=1… |
| CS-4 | Write access to the signal tree shall be authorised per client. A consumer shall not be able to publish, and no client shall write outside its declared signal set. | none | T | Implemented | Token scopes from scripts/dev_pki.py: consumers read only; each provider may provide only the signals its source declares. Suite check transport-security… |
| CS-5 | Signal values received from any provider shall be range- and type-validated against the catalogue before display, so a compromised provider cannot drive arbitrary content onto the cluster. | none | T | Implemented | Broker refuses out-of-catalogue values (suite check range-refusal); both HMIs read the catalogue restrictions from the broker at connect and refuse values… |
| CS-6 | Camera frames shall not leave the device and shall not be persisted. Only derived state, such as the drowsiness level, shall be published. | US-14 | I | Implemented | emulator/dms.py publishes six derived signals. Suite check dms-privacy traces every file and socket call for 12 s. Its first run with the monitor actually… |
| CS-7 | Security-relevant events, including authentication failure, unauthorised write attempt and client disconnection, shall be logged with a timestamp and without personal data. | none | T | Planned S3 | Section 5 |
| CS-8 | All third-party dependencies shall be pinned and inventoried as a software bill of materials, and the inventory shall be checkable against published vulnerabilities. | none | T | Implemented | scripts/sbom.py writes a CycloneDX inventory of 85 components across Python, Dart, Android and the broker image; suite check sbom fails on drift or on any… |
| CS-9 | No credential, key or certificate shall be committed to the repository; the development configuration shall use clearly marked non-production material. | none | T | Implemented | Suite check secrets; the development CA, keys and tokens are generated into git-ignored .run/pki/ with NOT FOR PRODUCTION in every subject, never committed |
| CS-10 | An update concept shall be described for the cluster and monitor software. It shall cover package authenticity and the behaviour on a failed update, as UNECE R155 and R156 require for type approval. | none | I | Planned S4 | Section 6 outline |
| CS-11 | Cybersecurity requirements shall be verified as part of the sprint regression suite, not as a separate end-of-project activity. | none | T | Implemented | 16-check suite in scripts/regression.sh, run before each demo and on every push by .github/workflows/regression.yml; each check names the requirements it… |

# 4. Story to requirements

| Story | Statement | Needs | Requirements | Verification of those requirements |
| --- | --- | --- | --- | --- |
| US-1 | As an HMI developer, I want every vehicle value available as a named VSS signal, so that I never touch bus formats or… | SN-3 | FR-1, FR-11 | I, T |
| US-2 | As a platform engineer, I want domain signals added as VSS overlays, so that we stay compatible with the upstream… | SN-2 | FR-2, FR-3 | I, T |
| US-3 | As an integrator, I want producers and consumers to meet only at the broker, so that either side can be swapped… | SN-3 | FR-5, FR-8 | I, T |
| US-4 | As a demonstrator, I want a realistic drive cycle without a vehicle, so that the cluster behaves plausibly on a desk. | SN-4 | FR-6 | D |
| US-5 | As an evaluator, I want the same cycle to replay identically, so that measurements of the two HMIs are comparable. | SN-1 | FR-7, NFR-7 | T |
| US-6 | As a driver, I want speed, RPM, gear, fuel and tyre pressures at a glance, so that I can read the vehicle state in… | SN-4 | FR-9, NFR-2 | D |
| US-7 | As a race engineer, I want lap, DRS and ERS state on the cluster, so that motorsport telemetry is visible live. | SN-4 | FR-10 | D |
| US-8 | As a stakeholder, I want the same design built in Flutter and Compose from one signal source, so that the platform… | SN-1, SN-2 | FR-12, FR-13 | D, I |
| US-9 | As a presenter, I want the cluster full-screen on the strip display but windowed elsewhere, so that the demo works on… | SN-4 | FR-14, IR-3, IR-4 | D |
| US-10 | As the sponsor, I want application cost separated from platform cost, so that the toolkit decision and the hardware… | SN-1 | FR-15, FR-16, FR-17, NFR-3 | A, T |
| US-11 | As a new engineer, I want to bring the stack up from documentation alone, so that the competence outlives the project. | SN-5 | NFR-5 | T |
| US-12 | As a driver, I want the cluster to warn me when I am drowsy or distracted, so that I regain attention before anything… | SN-7 | FR-18, FR-19, FR-21 | D, T |
| US-13 | As a safety engineer, I want sustained drowsiness to trigger a defined minimum-risk manoeuvre with a recovery window… | SN-7 | FR-20, SR-7 | D, T |
| US-14 | As a privacy officer, I want camera frames to stay on the device, so that driver monitoring never becomes surveillance. | SN-6 | CS-6 | I |
| US-15 | As a safety manager, I want an item definition, hazard analysis and safety-relevant signal list for the cluster, so… | SN-6 | SR-1, SR-2, SR-3, SR-11 | I |
| US-16 | As a security manager, I want a threat analysis and trust-boundary description for the signal path, so that the… | SN-6 | CS-1, CS-2, CS-3 | I, T |
| US-17 | As a reviewer, I want every requirement traced to a story and a verification, so that I can see what the prototype… | SN-6 | SR-9 | T |

# 5. Requirements not reached by a user story

These are traced only through their owning document and its verification column. Add a story or accept the gap at review.

| ID | Requirement | Owner | Status (W3) |
| --- | --- | --- | --- |
| CS-4 | Write access to the signal tree shall be authorised per client. A consumer shall not be able to publish, and no client shall write outside its declared signal set. | PULSE-SEC-001 | Implemented |
| CS-5 | Signal values received from any provider shall be range- and type-validated against the catalogue before display, so a compromised provider cannot drive arbitrary content onto the cluster. | PULSE-SEC-001 | Implemented |
| CS-7 | Security-relevant events, including authentication failure, unauthorised write attempt and client disconnection, shall be logged with a timestamp and without personal data. | PULSE-SEC-001 | Planned S3 |
| CS-8 | All third-party dependencies shall be pinned and inventoried as a software bill of materials, and the inventory shall be checkable against published vulnerabilities. | PULSE-SEC-001 | Implemented |
| CS-9 | No credential, key or certificate shall be committed to the repository; the development configuration shall use clearly marked non-production material. | PULSE-SEC-001 | Implemented |
| CS-10 | An update concept shall be described for the cluster and monitor software. It shall cover package authenticity and the behaviour on a failed update, as UNECE R155 and R156 require for type approval. | PULSE-SEC-001 | Planned S4 |
| CS-11 | Cybersecurity requirements shall be verified as part of the sprint regression suite, not as a separate end-of-project activity. | PULSE-SEC-001 | Implemented |
| FR-4 | A central broker shall serve the signal tree and allow multiple independent clients to subscribe concurrently. | PULSE-SyRS-001 | Implemented |
| FR-22 | The system shall accept an autonomy-stack provider that publishes planned trajectory, detected objects and drive mode into the broker as VSS signals, so that the cluster can render an autonomy state… | PULSE-SyRS-001 | Planned S3 |
| IR-1 | Client-to-broker communication shall use a language-neutral RPC interface supporting streaming subscriptions. | PULSE-SyRS-001 | Implemented |
| IR-2 | Signal semantics (units, ranges, allowed enumeration values, and encodings) shall be defined in the signal catalogue, not in client code. | PULSE-SyRS-001 | Implemented |
| IR-5 | The system shall run without transport security in the local development configuration, and the security posture shall be an explicit, documented choice. | PULSE-SyRS-001 | Implemented |
| NFR-1 | Responsiveness: displayed values shall track the telemetry source with no perceptible lag at a glance. | PULSE-SyRS-001 | Implemented |
| NFR-4 | Portability: migration to a different SoC or to a Yocto/AGL image shall not require HMI rework. | PULSE-SyRS-001 | Planned S3 |
| NFR-6 | Openness: the stack shall be built from open-source components, with no proprietary runtime licence required for evaluation. | PULSE-SyRS-001 | Implemented |
| NFR-8 | Maintainability: upstream components shall be consumed at pinned versions, not floating heads. | PULSE-SyRS-001 | Implemented |
| SR-4 | The system shall detect loss of a safety-relevant signal, including broker disconnection and stale values, and shall indicate the degraded state rather than continuing to display the last known value. | PULSE-SAF-001 | Implemented |
| SR-5 | Every safety-relevant signal shall carry a freshness criterion; a value older than its criterion shall be treated as invalid. | PULSE-SAF-001 | Implemented |
| SR-6 | Safety-relevant display elements shall fail visibly. A rendering fault shall not present a plausible but wrong value. | PULSE-SAF-001 | Planned S3 |
| SR-8 | Freedom from interference shall be argued for the mixed-criticality cockpit: infotainment content shall not be able to degrade or obscure the safety-relevant cluster surface. | PULSE-SAF-001 | Planned S4 |
| SR-10 | Any decision that would block a later ASIL argument, such as the choice of graphics stack, operating system or toolkit, shall be recorded as a documented deviation with its rationale. | PULSE-SAF-001 | Implemented |

# 6. Consistency

Needs: 7. Stories: 17. Requirements: 57. Dangling references: 0.