# PULSE Instrument Cluster

System Architecture Description

Ahmed Abdelghany

2026-09-23

Table of Contents

# Document control

| Field | Value |
| --- | --- |
| Document ID | PULSE-SAD-001 |
| Title | System Architecture Description |
| Version | 1.4 |
| Status | Baseline for sprint 1 review |
| Date | 2026-10-02 |
| Author | Ahmed Abdelghany |
| Reviewer | Cockpit Electronics / HMI Platform Group (sponsor) |
| Review gate | Sprint 1 review, week 3, 4 September 2026 |
| Parent | PULSE-SyRS-001 |
| Related | PULSE-StRS-001, PULSE-SAF-001, PULSE-SEC-001, PULSE-RTM-001 |

## Change log

| Version | Date | Author | Change |
| --- | --- | --- | --- |
| 1.0 | 2026-09-03 | A. Abdelghany | First baseline, written from the running system: context, components, interfaces, runtime, deployment, decisions and deviations, signal contract appendix |
| 1.1 | 2026-09-03 | A. Abdelghany | Layering, worked signal path (new section 5.4), runtime loop, manoeuvre state machine and deployment diagrams added; architecture principle diagram moved to draw.io |
| 1.2 | 2026-09-08 | A. Abdelghany | Section 5.4 worked example now traces one road speed from the bus to the dial with real signal names and values. DD-10 closed: the databroker image is pinned by tag and digest. Glossary pointer added |
| 1.3 | 2026-09-23 | A. Abdelghany | Broker interface now TLS with token authorisation; DD-6 and DD-11 closed; DD-12 records token authentication in place of client certificates; broker access module and CAN feeder spike added to the component catalogue |
| 1.4 | 2026-10-02 | A. Abdelghany | PULSE DBC and CAN overlay: every signal the cluster shows can now arrive over the bus (50 mapped signals in 14 frames). CAN feeder row and overlay table updated |

# 1. Purpose and scope

This document describes the architecture that satisfies PULSE-SyRS-001. It is the architecture baseline named in work package 01. It records what exists at week 3, what is planned, and the design decisions a later safety or security argument would need to know about (PULSE-SAF-001 SR-10).

It is a system-level description. Per-component software architecture (SWAD) is deferred to sprint 2 for the cluster and the providers.

Abbreviations used here are defined in the glossary, PULSE-StRS-001 section 8.

# 2. Architectural principle

Every component meets every other component only at the signal broker, and only through named signals from a standard catalogue. No component knows the others exist. Above the broker there is no CAN, no DBC and no hardware. Below the broker there is no user interface.

Architecture principle: providers publish, consumers subscribe, nothing meets anywhere but the broker.

Source: docs/diagrams/pulse-architecture.drawio (edit in draw.io, re-export with scripts/build-docs.sh).

Swapping the simulator for a real CAN bus changes one box on the left. No consumer is recompiled (FR-5, FR-8).

# 3. Context

## 3.1 System boundary

| Inside the boundary | Outside the boundary |
| --- | --- |
| Signal catalogue and overlays | The vehicle and its buses (no vehicle exists on the desk rig) |
| Databroker | The driver (observed by the camera, addressed by the display) |
| Telemetry providers: simulator, driver monitor, later CAN feeder and Autoware bridge | The display panel hardware |
| Both HMI applications | The host operating system, container runtime and graphics stack |
| Orchestration scripts and the regression suite | The Android Automotive emulator or device |

## 3.2 External interfaces

| Interface | Direction | Technology | Notes |
| --- | --- | --- | --- |
| Camera | In | V4L2 (/dev/video0), 1280 x 720 at 30 fps | Frames never leave the driver-monitor process (CS-6) |
| Strip display | Out | DisplayPort alt mode over USB-C, 2560 x 720 | Located at runtime by xrandr, placement via CLUSTER_GEOM (IR-4) |
| Vehicle bus | In | SocketCAN (vcan0 then a CAN HAT on SPI) | Planned S3 |
| Autonomy stack | In | Autoware planning simulation via ROS 2 bridge | Planned S3 to S4 |
| Android surface | Out | ADB reverse to reach the broker on the host | Emulator today; device is backlog |

# 4. Components

## 4.1 Component catalogue

| Component | Role | Implementation | Provenance | Location |
| --- | --- | --- | --- | --- |
| Signal catalogue | Names every signal, its type, unit, range and allowed values | COVESA VSS 6.0 plus AGL overlay plus two PULSE overlays, compiled with vss-tools to JSON | Upstream (VSS, AGL overlay), authored (PULSE overlays) | vss/ |
| Databroker | Serves the tree, validates writes, fans out subscriptions | Eclipse KUKSA databroker 0.7.0, Rust, in a container | Upstream | scripts/start-databroker.sh |
| Vehicle simulator | Publishes a plausible, correlated drive cycle; runs the minimum-risk manoeuvre in cruise mode | Python, kuksa-client 0.5.2 | Authored | emulator/telemetry_sim.py |
| Driver monitor | Face landmarks to fatigue, distraction and alert state; publishes derived state only | Python, MediaPipe FaceLandmarker (3.7 MB model, CPU), OpenCV | Authored (pipeline), upstream (model) | emulator/dms.py, emulator/models/ |
| Driver monitor viewer | Debug window: landmarks, metrics, thresholds; can stand in for the monitor | Python, OpenCV | Authored | emulator/dms_viewer.py |
| Engine audio | RPM-driven engine sound, a third independent subscriber | Python | Authored | emulator/engine_audio.py |
| Broker access | One module every Python client connects through: TLS, its role’s token, or plaintext on request | Python | Authored | emulator/broker.py |
| CAN feeder | KUKSA’s CAN provider on a virtual bus with AGL’s DBC plus the PULSE DBC; fed by the simulator’s CAN mode or by hand from the Vehicle Control panel | Container, Python, Tk | Upstream (provider), authored (feeders, panel, PULSE DBC) | emulator/can_bus.py, emulator/vehicle_control/, vss/dbc/ |
| Flutter cluster | The HMI on Linux; two screens (PULSE racing, classic analogue) in a swipeable pager | Flutter, Dart, custom-painted canvas; gRPC stubs generated from the KUKSA protos | Authored (UI, service), generated (stubs), scaffold (runner, one deliberate edit) | pulse-cluster/ |
| Compose cluster | The same design on Android Automotive | Kotlin, Jetpack Compose, gRPC | Authored | pulse-cluster-android/ |
| AGL reference cluster | AGL’s own Flutter cluster, used as a comparison point | Flutter | Upstream, cloned from AGL Gerrit, not part of this repository | flutter-instrument-cluster/ |
| Orchestration | Start and stop the stack, place the window, fall back to a normal window | Bash | Authored | run.sh, scripts/ |
| Documentation | Onboarding, this specification set, traceability generator | Markdown, Python | Authored | README.md, ONBOARDING.md, docs/ |

The two HMIs share no code. They share only the signal contract in appendix A.

## 4.2 Layering

Three layers and the abstraction line. Nothing above the line knows a CAN frame, a DBC file or a device.

Source: docs/diagrams/layering.drawio.

| Layer | Contents | Rule |
| --- | --- | --- |
| 03 Application | Flutter cluster, Compose cluster, driver monitor, safety-loop logic in the simulator, Autoware bridge, vehicle simulator | Consumes and produces signals only |
| Abstraction line |  | No CAN, DBC or device knowledge above this line |
| 02 HAL and middleware | KUKSA databroker, VSS tree and overlays, gRPC transport, CAN provider with DBC decode, MediaPipe runtime, window system | Frames become signals here |
| 01 MCAL and board support | Linux kernel and device tree, SPI and CAN controller driver, SocketCAN, V4L2 camera, DRM/KMS graphics | Silicon and kernel |

# 5. Interfaces

## 5.1 Broker interface

| Property | Value |
| --- | --- |
| Protocol | gRPC over HTTP/2 |
| API | kuksa.val.v1 (Get, Set, Subscribe streaming) |
| Address | 127.0.0.1:55555, bound to loopback only |
| Transport security | TLS to a development certificate by default; plaintext only with PULSE_INSECURE=1 and a warning (PULSE-SEC-001 CS-3) |
| Authorisation | Signed token per client, scoped: consumers read, providers provide their own signals (PULSE-SEC-001 CS-4, DD-12) |
| Validation | Broker rejects a value whose datatype, allowed list or min/max does not match the catalogue (IR-2, CS-5) |
| Client stubs | Dart and Kotlin generated from the same .proto files; Python via kuksa-client |

kuksa.val.v1 was chosen over the newer kuksa.val.v2 because AGL’s reference stack and the AGL overlay target v1. Migration to v2 is a recorded decision, section 8.

## 5.2 Publish rates

| Producer | Signals | Rate |
| --- | --- | --- |
| Vehicle simulator, fast group | Speed, engine speed, gear, throttle, lap time, DRS, ERS, intervention state | 10 Hz, changed values only |
| Vehicle simulator, slow group | Fuel, coolant, odometer, tyre pressures, heart rate | 1 Hz |
| Vehicle simulator, static | Units, performance mode, lamp states | Once at start |
| Driver monitor | Fatigue, distraction, attention, eyes-on-road, active, alert state | 10 Hz publish, 15 Hz processing |

## 5.3 Signal contract

The signal contract is the set of VSS paths a consumer may rely on. It is listed in appendix A and is the only coupling between producers and consumers. A signal contract test (sprint 2) asserts that every path in appendix A resolves in the compiled tree with the expected datatype and unit.

## 5.4 Worked example: one signal, bottom to top

Follow one value, a road speed of 87.4 km/h, from the wire to the dial. It travels as Vehicle.Speed, which appendix A defines as a float in km/h. Today the simulator enters at step 3; steps 1 and 2 are the sprint 3 CAN provider.

| Step | What happens | What the value is here |
| --- | --- | --- |
| 1 | The powertrain ECU puts the frame carrying PT_VehicleSpeed on the bus. SocketCAN hands the raw bytes to user space | Raw bus bytes, meaningless on their own |
| 2 | The provider applies the scale and offset the DBC defines for that signal, then maps it to a catalogue path. The mapping and its 100 ms interval are in vss/agl_vss_overlay.vspec | A number, 87.4, now named Vehicle.Speed |
| 3 | The provider publishes to the broker over gRPC. Today emulator/telemetry_sim.py sets the same path at 10 Hz instead | Vehicle.Speed = 87.4 |
| 4 | The broker checks it against the catalogue entry. A wrong type or an out-of-range value is refused and never reaches a consumer (IR-2, CS-5) | float, km/h, in range, accepted |
| 5 | Every open subscription is pushed the value with its timestamp | Flutter and Compose both redraw the dial |

Only steps 1 and 2 know that CAN exists. From step 3 on the value is just a named number, and a consumer cannot tell whether it came from a bus, the simulator or a replayed log. Note also where the protocols stop: gRPC exists only above the line, and below it there is no RPC at all.

One signal from the wire to the dial. Steps 01 and 02 are the sprint 3 CAN provider; today the simulator enters at step 03.

Source: docs/diagrams/signal-path.drawio.

# 6. Runtime view

One turn of the loop, repeated at the broker tick:

One turn of the cycle. It never stops turning, and no central coordinator drives it.

Source: docs/diagrams/runtime-loop.drawio.

| Step | Name | What happens |
| --- | --- | --- |
| 01 | Acquire | CAN frames off the bus (simulator today), camera frame off V4L2 |
| 02 | Interpret | DBC decode on the vehicle side; face landmarks and behaviour metrics on the driver side |
| 03 | Publish | Both providers write VSS signals at 10 Hz; the broker range-checks each value against the catalogue |
| 04 | Distribute | The broker pushes each change down every open subscription stream; no provider knows who listens |
| 05 | Decide | The simulator’s intervention state machine evaluates the alert state |
| 06 | Render | Both clusters redraw: values, warnings, the alert banner, the countdown |
| 07 | Actuate | Hazards on, cruise disengaged, controlled braking to standstill (simulated) |
| 08 | Feed back | The actuation appears as new signal data and the loop closes without a central coordinator |

Perception to displayed alert stays inside one 100 ms broker tick. Cluster and monitor never call each other.

## 6.1 Minimum-risk manoeuvre state machine

Held in the simulator’s cruise mode, driven only by Vehicle.Driver.Monitoring.AlertState:

The minimum-risk manoeuvre state machine with its current parameters.

Source: docs/diagrams/mrm-state-machine.drawio.

| State | Entry | Exit |
| --- | --- | --- |
| NONE | Start, or countdown aborted, or resume complete | AlertState becomes DROWSY: go to COUNTDOWN |
| COUNTDOWN | DROWSY sustained past the 3 s alert hold | AlertState leaves DROWSY: back to NONE. Countdown of 5 s expires: go to BRAKING with hazards on |
| BRAKING | Countdown expired | Speed reaches 0: go to STOPPED |
| STOPPED | Standstill | AlertState NONE for 5 s: hazards off, resume cruise, back to NONE |

The specification of this mechanism as a safety mechanism, with its parameters, is PULSE-SAF-001 SM-3.

# 7. Deployment view

The desk rig at week 3 and the sprint 3 target rig. The same Flutter bundle runs on both; the embedder is the only platform-specific layer.

Source: docs/diagrams/deployment.drawio.

## 7.1 Desk rig, week 3

| Node | Hosts | Notes |
| --- | --- | --- |
| Development PC, Ubuntu, x86-64 | Databroker (container), simulator, driver monitor, engine audio, Flutter cluster | One process each; started by run.sh or scripts/start-stack.sh |
| Corsair Xeneon Edge | Flutter cluster surface | 2560 x 720, DisplayPort alt mode; located at runtime |
| USB camera | Driver monitor input | 1280 x 720 at 30 fps |
| Android Automotive emulator | Compose cluster | Reaches the host broker through ADB reverse |

## 7.2 Target rig, sprint 3

| Node | Hosts | Notes |
| --- | --- | --- |
| Raspberry Pi 5, 8 GB | The whole Linux stack | Establishes the performance envelope off the desktop (NFR-3) |
| CAN HAT on SPI | MCP2515-class controller with transceiver | Exposed as SocketCAN; vcan0 first |
| Corsair Xeneon Edge, USB camera | As above |  |

Nothing on either rig is automotive-qualified, and the software does not assume it is.

# 8. Design decisions and deviations

Recorded here because PULSE-SAF-001 SR-10 requires every decision that could block a later ASIL argument to be written down with its rationale.

| ID | Decision | Rationale | Consequence for a production programme |
| --- | --- | --- | --- |
| DD-1 | HMI toolkit: Flutter on Linux as the primary path | AGL’s cluster toolkit of choice; open source; one engineer can reach it | No safety-qualified Flutter renderer exists; a production cluster would need a qualified renderer or a supervisor that verifies the drawn output (SR-6) |
| DD-2 | Second HMI: Jetpack Compose on Android Automotive | Answers the sponsor’s question with a measured pair | AAOS platform footprint is 25x the Linux path; Linux stays primary |
| DD-3 | Broker: Eclipse KUKSA databroker in a container | Reference implementation of the VSS broker; used by AGL | Container runtime is a desk convenience; target deployment runs it as a native service |
| DD-4 | API version: kuksa.val.v1 | Matches AGL and its overlay | Migration to v2 when AGL moves; stubs regenerate from the protos |
| DD-5 | Signal catalogue pinned to VSS 6.0 | Constraint C-2 | Upgrades are reviewed changes; the AGL overlay must be re-validated per version |
| DD-6 | Development transport: plaintext gRPC on loopback (--insecure) | Desk rig, one host | Closed in sprint 2: secure by default, plaintext only on request with a warning |
| DD-7 | Window system on the desk rig: X11 with xrandr placement | Development host convenience | Target uses Wayland; the runner reads CLUSTER_GEOM so placement logic does not change |
| DD-8 | Driver monitor runs on CPU, no accelerator | 6.5 ms per frame is already inside budget; avoids hardware dependence (A-2) | A production DMS would still need an infrared camera and a qualified evaluation set (backlog) |
| DD-9 | Safety-loop actuation lives in the simulator | No vehicle exists; the manoeuvre must be demonstrable | On a real vehicle the actuation moves to a motion controller; the trigger signal contract stays |
| DD-10 | Databroker image consumed at tag latest | Convenience at project start | Closed: pinned to the 0.7.0 tag and digest in scripts/start-databroker.sh |
| DD-11 | Flutter SDK version not pinned | Convenience at project start | Closed in sprint 2: 3.47.0 in .fvmrc, floor in pubspec.yaml, enforced by the toolchain-pin check |
| DD-12 | Clients authenticate with a signed token over TLS, not a client certificate | KUKSA 0.7.0 cannot verify client certificates | Meets CS-3’s goal by a different mechanism; the requirement text or the broker changes later. See PULSE-SEC-001 section 5 |

# 9. Verification approach

| Level | What | When |
| --- | --- | --- |
| Signal contract test | Every path in appendix A resolves in vss_agl.json with the expected type and unit; overlay paths present | Sprint 2, then every demo |
| Provider range test | Simulator and driver-monitor outputs stay inside catalogue ranges over a full cycle | Sprint 2 |
| Broker smoke test | Broker up, tree served, a subscribe stream receives updates within one tick | Sprint 2, then every demo |
| UI smoke test | Each HMI launches, connects, and renders a known value | Sprint 2 |
| HMI purity check | No CAN or DBC token in HMI source (FR-11) | Sprint 2 |
| Benchmark | CPU, memory, artefact size under the replayed cycle, both HMIs | Sprint 4 |
| Clean-machine bring-up | From README and ONBOARDING.md only | Done 2026-08-18; repeated at handover |

# Appendix A. Signal contract

## A.1 Signals consumed by the HMIs

| VSS path | Datatype | Unit | Source | Notes |
| --- | --- | --- | --- | --- |
| Vehicle.Speed | float | km/h | Simulator | Primary value |
| Vehicle.Powertrain.CombustionEngine.Speed | uint16 | rpm | Simulator | Redline band, shift ghost |
| Vehicle.Powertrain.Transmission.SelectedGear | int8 |  | Simulator | -1 = R, 0 = N, 126 = P, 127 = D |
| Vehicle.Powertrain.Transmission.PerformanceMode | string |  | Simulator | Allowed values uppercase |
| Vehicle.Powertrain.FuelSystem.RelativeLevel | uint8 | percent | Simulator |  |
| Vehicle.Powertrain.FuelSystem.Range | uint32 | m | Simulator |  |
| Vehicle.TraveledDistance | uint32 | m | Simulator | Odometer |
| Vehicle.Chassis.Axle.Row1.Wheel.Left.Tire.Pressure | uint16 | kPa | Simulator | Front left |
| Vehicle.Chassis.Axle.Row1.Wheel.Right.Tire.Pressure | uint16 | kPa | Simulator | Front right |
| Vehicle.Chassis.Axle.Row2.Wheel.Left.Tire.Pressure | uint16 | kPa | Simulator | Rear left |
| Vehicle.Chassis.Axle.Row2.Wheel.Right.Tire.Pressure | uint16 | kPa | Simulator | Rear right |
| Vehicle.Body.Lights.Hazard.IsSignaling | boolean |  | Simulator | Set during the manoeuvre |
| Vehicle.Driver.HeartRate | uint16 | bpm | Simulator |  |
| Vehicle.Driver.AttentiveProbability | float | percent | Driver monitor |  |
| Vehicle.Driver.FatigueLevel | float | percent | Driver monitor |  |
| Vehicle.Driver.DistractionLevel | float | percent | Driver monitor |  |
| Vehicle.Driver.IsEyesOnRoad | boolean |  | Driver monitor |  |
| Vehicle.Driver.Monitoring.IsActive | boolean |  | Driver monitor | Overlay |
| Vehicle.Driver.Monitoring.AlertState | string |  | Driver monitor | Overlay; NONE, DROWSY, DISTRACTED, NO_FACE |
| Vehicle.Driver.Monitoring.InterventionState | string |  | Simulator | Overlay; NONE, COUNTDOWN, BRAKING, STOPPED |
| Vehicle.Driver.Monitoring.InterventionCountdown | uint8 | s | Simulator | Overlay; 0 when idle |
| Vehicle.Motorsport.LapNumber | uint16 |  | Simulator | Overlay |
| Vehicle.Motorsport.LapTime.Current | float | s | Simulator | Overlay |
| Vehicle.Motorsport.LapTime.Best | float | s | Simulator | Overlay; 0 until a lap completes |
| Vehicle.Motorsport.DRS.IsAvailable | boolean |  | Simulator | Overlay |
| Vehicle.Motorsport.DRS.IsActive | boolean |  | Simulator | Overlay |
| Vehicle.Motorsport.ERS.Level | uint8 | percent | Simulator | Overlay; max 100 |
| Vehicle.Motorsport.ERS.State | string |  | Simulator | Overlay; DEPLOY, HARVEST, IDLE |

## A.2 Overlay files

| File | Adds | Origin |
| --- | --- | --- |
| vss/agl_vss_overlay.vspec | AGL’s cockpit signals: steering-wheel switches, infotainment, navigation, acceleration, angular velocity | AGL meta-agl-demo, vss-agl_6.0 |
| vss/pulse_vss_overlay.vspec | Vehicle.Motorsport branch: lap number, lap times, DRS, ERS | PULSE |
| vss/dms_vss_overlay.vspec | Vehicle.Driver.Monitoring branch: active, alert state, intervention state, countdown | PULSE |
| vss/pulse_can_overlay.vspec | Bus mappings only, no new paths: fuel from AGL’s PT_FuelLevelPct; ERS, DRS, tyre pressures and driver monitoring from vss/dbc/pulse.dbc (frames 0x210, 0x220, 0x300, 0x400, clear of every AGL ID) | PULSE |

## A.3 Regeneration

The compiled tree is regenerated from the pinned VSS 6.0 specification and the three overlays by the command in the README section “Regenerate the VSS JSON”. Regeneration has been confirmed byte-identical (FR-3).